SHELF-LIFE JUSTIFICATION & VALIDATION PLAN

Gevriesdroogd B.V.
Noodrantsoen.nl (handelsnaam)
Zwartewaterallee 56
8031 DX Zwolle
Nederland

KVK: 83720642
BTW: NL862969050B01
EORI: NL862969050

1. Product Description

Product: Long-Life Purified Drinking Water

The product: ‘50 jaar houdbaar drinkwater (8 Liter)’

https://noodrantsoen.nl/product/noodrantsoen-drinkwater-50-jaar-houdbaar-tray-24blikken-8liter/

- Packaging: Hermetically sealed aluminium cans with internal food-grade protective lining

- 24 pcs in carton box totalling 8 Liters

- 63 boxes on non-stackable euro pallets 80x120cm (height 120cm)

- Declared Shelf Life: Up to 50 years under specified storage conditions (1 – 45 °Celsius)

2. Technical Basis for Long-Term Stability

The declared extended shelf life is based on the following scientifically recognized principles:

2.1 Water Stability

Heat treated Purified water with very low Total Dissolved Solids (TDS < 2 ppm) contains minimal dissolved minerals and organic matter.

This significantly reduces:

- Microbial growth potential

- Chemical reactivity

- Precipitation or degradation reactions

Water itself is chemically stable when protected from contamination and light.

2.2 Microbiological Control

Production takes place under a HACCP-based food safety management system in accordance with Regulation (EC) 852/2004.

The process includes:

- Multi-stage purification

- Heating up to 90 degree celcius

- Controlled hermetic  hygienic cooling down

- Controlled hygienic filling

- Immediate hermetic sealing

Microbiological safety of shelf-stable beverages is primarily dependent on:

- Absence of viable microorganisms at filling

- Integrity of the hermetic seal

- Prevention of post-process contamination

When these conditions are met, microbial growth cannot occur due to lack of contamination and absence of nutrients.

2.3 Packaging Barrier Properties

The product is packed in aluminum cans with an internal food-grade lining.

Aluminum packaging provides:

- Complete light barrier

- High oxygen barrier

- Structural integrity

- Long-term dimensional stability

Under normal and foreseeable storage conditions, the packaging protects the contents from external contamination and environmental degradation.

Packaging materials comply with Regulation (EC) 1935/2004 and are manufactured in accordance with Regulation (EC) 2023/2006 (Good Manufacturing Practice).

3. Shelf-Life Rationale

The declared shelf life of up to 50 years is based on:

- Chemical stability of purified water

- Absence of nutrients supporting microbial growth

- Hermetically sealed aluminum packaging

- Barrier protection against light and oxygen

- Controlled storage conditions

The shelf life assumes:

- Storage between 1°C and 45°C

- Dry conditions

- No mechanical damage to packaging

- No exposure to extreme temperatures or corrosive environments

4. Validation & Ongoing Stability Program

To ensure continued compliance and safety, Gevriesdroogd B.V. maintains the following validation framework:

4.1 Batch Release Testing

Each production batch is subject to:

- Microbiological testing (Total viable count, relevant water hygiene indicators)

- Visual inspection

- Seal integrity control

Certificates of Analysis (COA) are available per batch.

4.2 Retained Sample Program

Production batches are retained and stored under controlled conditions for periodic evaluation.

Periodic testing may include:

- Microbiological analysis

- Sensory evaluation

- Packaging integrity inspection

4.3 Packaging Compliance

Declarations of Compliance from packaging suppliers are maintained, confirming:

- Food contact suitability

- Compliance with EU 1935/2004

- Suitability for long-term contact with drinking water

5. Statement of Responsibility

The declared shelf life represents the manufacturer’s technical assessment based on:

- Process control

- Packaging characteristics

- Product composition

- Food safety management system

This declaration does not replace ongoing verification testing but reflects the expected stability of the product under defined storage conditions.

Issued under the sole responsibility of Gevriesdroogd B.V.

Signed,

Marielle Seubers

Directeur - CEO
Gevriesdroogd B.V.
12-03-2025